PTSM FPGA Design for 8 Data lines

Hartmut Sadrozinski February 4, 2002

Cables

DAQ

PTSM

DSSD

PMFE:

AMP/DIS

MCM

ROC:

FPGA

The PTSM will consist of a double-sided silicon strip detector (DSSD), an frontend ASIC Particle Microscope Front End PMFE, a readout controller ROC (FPGA) and external DAQ.

## Frontend ASIC PMFE

The readout ASIC will be a revision of the analog GLAST99 GTFE, to include a switch to allow double-sided readout.  It only discriminates the hits, latches them and clocks 8 of them out onto 8 data lines. The calibration buses and the threshold are common to all PMFE. Calibration is a common bus 4 wide feeding all PMFE chips. Every 4th chip gets the same input.

PTSM

Shaping time	[s]		~0.2

Peaking time 	[s]		<0.25

TOT LET Range [fC]		0.8 – 300 for both p and n

TOT saturation time [s]	<30

Fast OR output		None

# of TOT OR 			None

Polarity of signal		Both + and -, switchable

Noise	[e]			<1,000 (6cm SSD)

Threshold [e]			5,000

Threshold [fC]			0.8

RO Frequency [MHz]		>64	(goal 100)

TOT Bucket [fC]		300*70/33,000=0.7

Output				Serial LVDS

Power / channel [mW]	~1

# of channels/ASIC		64

# of channels/data line	8

Width of Channel [m]	88

Channel Mask			None

Calibration			external

## RO Controller ROC	FPGA

We anticipate one ROC for 4 PMFE ASICs. The controller has to accept the fast OR trigger signals and make trigger decisions: for example only one chip hit, or coincidence on both sides etc. To allow to push the speed of the system, use >400MHz technology, and are targeting Virtex by Xilinx.

PTSM

Method			FPGA

Location			MCM

Clock Frequency [MHz]

External		64 ->100

Internal		>400?

Function			Make Trigger

Determine hit channel address

Determine Start TOT, Stop TOT

Assemble event

Set Calibration DAC and bus

Set THR DAC

RESET

Per event send doublet of data out per hit

Address, Start TOT, Stop TOT

#### PTSM ReadOut Architecture

The FPGA has three functional modes: DATA, CONTROL and CALIBRATION.

### DATA

##### PMFE

Amplify and discriminate collected charge 	-> output is digital pulse of TOT length

Latch continually all strips with 8MHz	-> Time stamp data frozen

Clock out continually data grouped into 8 channels with 64 MHz serially onto 8 data lines

##### ROC

Receive serial data on 8 data lines		->  Clock hits from one line into 8 channels

Generate fast trigger  (Mask)	-> 		Start TOT with rising edge

Determine N =Hit channels of event

(Send TRG to other FPGA s and CPU)

Increment TOT counter of hit channels

Stop TOT counter at first low count		->STOP TOT

Send N, Start TOT, Stop TOT to I/O		->Finish data for one event.

Ready for next event

### CONTROL

##### ROC

Propagate CLR

Set THR

### CALIBRATION

##### ROC

Labview sets DAC value

ROC Selects MUX to one of 4 CAL lines

Send STROBE

Read data etc