# Count Rate Analysis

Muon Lifetime Experiment: Design #2

Area of detector A = 15 x 75 cm2 = 1125 cm2

Area of detector B or C = 30 x 75 cm2 = 2250 cm2

Angle subtended by B or C ≈ /2 = 90

Muon Decay times are measured:

Start condition = A and not (B or C)

Stop condition = B or C or time-out (10 ms)

Once started, the clock continues to run until a stop is triggered or a time-out.  Second start signals, while the clock is running, are ignored.  A time output is generated only if a stop is triggered before time-out.   The Veto output for B or C takes about 30 ns.  The input to the logic gate for A should, therefore, be delayed by 30 ns and the data adjusted accordingly.

Muons of all energies:

N  = Expected flux of muons of any energy 	≈ 0.02 Hz/cm2

N(A) = Expected muon count rate through detector A	≈ 18 Hz

N (B) = Expected muon count rate through detector B or C	≈ 0.5 N (A)

Muons with E < 1 GeV

dN/dE = Expected muon count rate per energy	≈ 0.004 Hz/GeV cm2

dE/dx (paper) = energy loss rate for paper and E  50MeV	≈ 1.7 MeV/cm

Emax = max. muon energy that can be trapped in the cavity	≈ 40 MeV

Emin = min. Electron energy that can escape the cavity	≈ 20 MeV

f1 = Fraction of all muons with E < Emax 	≈ 0.010

f2 = Fraction of decay electrons with E > Emin 	≈ 0.7

Nd = Expected count rate of decays	≈ 0.13 Hz

A =  Discriminator pulse width for A	= 50 ns

B =  Discriminator pulse width for B	= 50 ns

C =  Discriminator pulse width for C	= 50 ns

Detector/discriminator efficiency measurements

Detector A:

TA = Threshold voltage	=     mV

SA = Singles rate 	=     Hz

CA = Coincidence efficiency [(A and B and C)/(B and C)]	=

Detector B:

TB = Threshold voltage	=     mV

SB = Singles rate 	=     Hz

CB = Coincidence efficiency [(A and B and C)/(A and C)]	=

Detector C:

TC = Threshold voltage	=     mV

SC = Singles rate 	=     Hz

CC = Coincidence efficiency [(A and B and C)/(A and B)]	=

## Possible timing events:

- While clock is stopped, A is triggered by:	[As ]

  - Random event in A only	False Start

  - Charged particle 		[N(A) ]

    - Accidental coincidence in AB or AC	Missed Start

    - Misses B or C	Missed Start

    - Continues through B or C

      - And is detected	-

      - Is not detected	False Start

    - Captured in chamber	Start

- While clock is running

  - Second Start signal is received

    - Accidental coincidence with False Start	[

    - Captured muon	[

  - No stop before time-out	-

  - Stop signal

    - Random event in B or C	[

    - Coincidence with second muon	[

    - Coincidence with decay of another muon	[

    - Muon decay detected	STOP